Martelco Equipment Hire  PLANT RISK ASSESMENT

TYPE OF PLANT- FRONT END LOADER

MAKE- VOLVO

MODEL- L70F

| HAZARD TYPES | LEVEL OF RISK | ACTION TO REMOVE HAZARD OR CONTROL RISK (as far as practicable) |
| --- | --- | --- |
| A. CRUSHING,ENTANGLEMENT,CUTTING, STABBING,SHEARING,FRICTION,STRIKING |  |  |
| CRUSHING/SHEARING/ ENTANGLEMENT | MODERATE | Guards are provided in accordance with plant code requirements for guarding. Guarding is of a fixed permanent nature which can only be removed with tools. A motion alarm is fitted to all Australian units to warn of any movement forwards or backwards. Only one pivot point exists, risk of crushing/ entanglement is low and warning decals are displayed. |
| CUTTING | LOW | See above |
| FRICTION | LOW | Operators are not subject to friction as there are no high-speed exposed parts. Mechanical failure due to friction is reduced with self lubricating bushes & wear pads. Friction points have grease nipples to facilitate lubrication. Engine lubrication points are easily accessed. |
| STRIKING | MODERATE | Plant is fitted with motion alarms to warn people when plant is moving. Hand brake and emergency stop levers are easily located and reached by operator. |
| CUTTING, STABBING | LOW | Contact surfaces such as handles and vehicle entry points have no sharp edges. All steps are non slip. |
| B. ERGONOMIC, SLIPPING, RIPPING, FALLING |  |  |
| SEATING | LOW | Seating is ergonomically designed with padding and shock absorbing springs. |
| EXCESSIVE EFFORT, BAD POSTURE | LOW | Controls are either of lever, toggle or button operation. Non-assisted controls are minimized using electrical actuation. Where controls are mechanical in nature operating effort is reduced as far as practicable. |
| OPERATING STRESS | LOW | Control panels use pictures for functions and levers which control direction operate in that direction. |
| LIGHTING | LOW | Units are fitted with front mounted lights and reversing lights to assist in operation |

| HAZARD TYPES | LEVEL OF RISK | ACTION TO REMOVE HAZARD OR CONTROL RISK (as far as practicable) |
| --- | --- | --- |
| C. HIGH TEMP OR PRESSURE, FIRE/ EXPLOSION |  |  |
| HIGH PRESSURE HYDRAULIC LINES | LOW | Hydraulic hoses have a bursting pressure several times that of the working pressure |
| HIGH TEMPERATURES | LOW | Hot surfaces are either shrouded or put out of reach. |
| D. SUFFOCATION/ DROWNING |  |  |
| SUFFOCATION/ DROWNING | N/A | Exhaust gasses are pointed away from operator. The size of the machine prevents use in confined spaces therefore exhaust gas inhalation is not deemed to pose a threat. |
| E. ELECTRICAL |  |  |
| ACCIDENTAL ELECRICAL SHOCK | LOW | Cables insulated and secured to plant. Major current carrying cables are color coded. Cables have protective rubber boots over connection points to prevent contact shorting during maintenance. |
| LOOSE WIRE SHORTS | LOW | Connectors used are either insulated crimp lugs, locking plastic plugs or permanent type clamps. Fuses are also fitted to main cables. |
| WORKING TO CLOSE TO POWER LINES | N/A | The height of machine is not enough to pose a threat to power lines. |
| F. STABILITY |  |  |
| STABILITY | HIGH | The plant is designed to be stable on flat surfaces, slight incline/ decline. Consult the safe operating procedures manual on grades. Roll over protection systems have been designed and fitted and seat belt must be worn at all times. |
| TRAVELLING HAZARDS ( e.g. rough surfaces) | MODERATE | Consult safe operating procedures manual. |
| G. STRUCTURAL FAILURE |  |  |
| STRUCTURAL FAILURE | MODERATE | Rigorous stress analysis and testing is used to ensure structural soundness before introduction to the workplace. Visual inspection is preformed on plant daily. |
| COMPONENT FAILURE DUE TO FATIGUE/ WEAR | MODERATE | Corrosive surfaces are painted; components subject to wear have provisions to minimize wear by using sacrificial components or lubrication. Components that are not self lubricating have grease nipples provided. |
| H. MAINTINANCE |  |  |
| ROUTINE INSPECTION OR MAINTINANCE | NIL | Components which require regular maintenance such as filters are placed in easily accessed areas. Inspection and maintenance procedures are placed in the SOP manual. |
| BATTERY CHARGING | LOW | Battery is automatically charged while engine is running, and as it is only being trickle charged, gas (hydrogen) build-up is not considered a problem. |
| GUARD REMOVAL | LOW | Guards are provided and are of fixed permanent nature. This can only be removed with tools. |

| HAZARD TYPES | LEVEL OF RISK | ACTION TO REMOVE HAZARD OR CONTROL RISK (as far as practicable) |
| --- | --- | --- |
| I. TRANSPORT |  |  |
| TRANSPORT | MODERATE | Recommended Tie down points are located and decals in place to ensure safe transport. When loading up ramp operator must ensure plant is in low speed (turtle) and must have a non- slip surface. Inspections on tie down points are regularly conducted. |
| UNINTENDED USE | MODERATE | Correct operating procedures are placed in the manual. Warning decals are placed on the machine, safety warnings are also placed in the SOP manual. Training should be conducted in the safe use of this machinery. |
| EXCESSIVE NOISE | LOW | Motor is under cover and within acceptable sound limits. |
| J. OCCUPATIONAL HAZARDS |  |  |
| DECAL OR SAFETY SIGN REMOVAL | HIGH | Decals have permanent type marking and weatherproof backing. Specification plate is stamped for longevity. Safety warnings are placed in manual. Annual inspection requires that decals are checked for readability and are in place |

This risk assessment should be used as a guide only, consult Operators Manual or Safe Operating Manual for any uncertainties

20/8/10

This risk assessment was carried out by Luke Martino